!! Important !!

/*To Up-date the Controller Code*/

There are 3 files to download the controller.  Follow these directions to accomplish this.  Please call if you want Helmel to walk you through this over the phone.

Quit GEOMET

Run Pwin

Do a ^K ^D in the communication window

File/Open navigate to the new plc3.pmc file and open it...( located in the Backups\PMAC_PLC folder on the Geomet CD )

Right click on the file and choose download...accept the default download settings.

After download is complete, close the message box and file.

For Mini-PMAC systems, repeat the above for the plc1.pmc file.

Type M5->*

Repeat the above for the Prog12.pmc file.

Type SAVE<enter> in the communication window.

Type ENABLE PLC 1<enter>  in the communication window

Close PWin

Run GEOMET....

Good luck!