!! Important !!

/*PMAC-Mini BUS mounted cards with LCD Joystick */

For proper communication to the PMAC card, the following steps must be performed before running Geomet version 5.00.000.001.

Quit GEOMET

Run Pwin

Do a ^K ^D in the communication window

File/Open navigate to the new plc6.pmc file and open it...( located in the Backups\PMAC_PLC folder on the Geomet CD )

Right click on the file and choose download...accept the default download settings.

After download is complete, close the message box and file.

Type SAVE<enter> in the communication window.

Type ENABLE PLC 1<enter>  in the communication window

Close PWin

Run GEOMET....

Good luck!